Current as of 1SEP2015

[image: image1.png]


 [image: image2.png]


[image: image3.png]



Porsche Club of America

Minimum Standard Drivers Education Technical Inspection Form
OktoberFiesta 2015

Driver #1 (Please Print): _______________________________________________________________ 


Emergency Contact Name & Phone: _______________________________________________
Driver #2 (Please Print): _______________________________________________________________

Emergency Contact Name & Phone: _______________________________________________
Tech Inspector’s Name (Please Print): ____________________________________________________
Car Make & Model: ____________________________ Color: __________________ Number: _______
ITEMS INSPECTED: 
_____ Helmet: Snell rated current or last date certificate, no cracks; chin strap in good condition. 

_____ Roll Bar & Cages: Mandatory for all open & semi-open cars without manufacturer’s structural or deploying roll over protection; roll bars must pass the broomstick rule and must be covered with closed cell foam padding in any place where a helmet could come in contact with the bar.  
_____ Seat Belts: Factory or better installation in both seats; must be securely mounted; belts not frayed; equal restraints for passenger and driver. 4-point harnesses prohibited except Schroth Quick Fit. 5-, 6- and 7-point harnesses must use seat with factory routing holes. 

_____ Structural Integrity: no loose body or interior parts; no structural rust at or near suspension points.   

_____ Doors: must be operational from inside and outside.  

_____ Windshield: no cracks, condition does not impair vision.   

_____ Mirrors: minimum of single mirror in good condition and securely mounted.  

_____ Gas Cap: tightly closed and no leakage. 

_____ Fluids: NO DRIPPING LEAKS of any fluid of any type. 

_____ Fluid Lines: proper fittings and line condition; no external clamps used on pressurized oil lines. 

_____ Battery: properly secured; no apparent corrosion. 

_____ Throttle Return: freely operating and proper spring(s). 

_____ Engine: proper exhaust in good condition; general condition of engine and accessories. 

_____ Belts: fan and other auxiliary belts in good condition with proper tension.
_____ Brakes: Sufficient brake pads (3/16 inch minimum); rotors and pads are in good condition; no cuts or abrasions in brake lines.  There should be no visible cracks extending to the edge of the rotor.  With co-driver, pads should exceed minimum standard.   

_____ Brake Fluid: Sufficient brake fluid; premium fluid recommended.  Brake fluid must be changed at least annually.  * Date brake fluid was last changed: ___/___/____ 

_____ Brake Lights: functional and visible brake lights. 

_____ Pedals: All pedals must have free return and be in good operating condition.  Brake pedal must be firm.   

_____ Wheels-Overall Condition: no cracks, bends or flaws; valve stems must have airtight caps.        

_____ Wheel Bolts & Lugs: all lug nuts engaged; pay special attention to after-market wheels & Turbo Twist wheels; if using track tires, steel lug nuts should be used.  

_____ Wheel Centerlocks: make sure locking devices are fully engaged.  

_____ Wheel Bearings: proper free rotation; no looseness in hub.   

_____ Front Suspension: no excessive play; ball joints and tie rods secure; boots and all seals in good condition. 

_____ Rear Suspension: half shaft boots in good condition; mounting hardware in good condition; no leakage.  
_____ Exhaust System: securely fastened and in good condition.  

Tech Inspector Signature:___________________________________ Date: _____________
THE UNDERSIGNED INDIVIDUAL(S) HEREBY CERTIFY THAT:

 

1.  This OFFICIAL TECHNICAL INSPECTION FORM (THE “FORM”) submitted to the  Carrera Region PCA (the “Region”) of the Porsche Club of America, in connection with the above noted High Performance Driver Education Event was performed on the date shown below by a Porsche mechanic or other person authorized by the Region to conduct such an inspection.

2.  The completed Form correctly and accurately shows the condition of the Car identified above on the date of the inspection.

3.  There have been no changes in the condition of the Car since the date of the inspection that would affect the track-worthiness of the Car.  The undersigned hereby agrees to defend, indemnify and hold harmless PCA, its zones and regions, as well as the inspector, from any and all claims or demands arising directly or indirectly from any incorrect and/or inaccurate statements set forth in this form.

 

I understand that the technical inspection performed on my Car and my helmet is solely for the purpose of meeting minimum standards of car preparation for the Zone’s or Region’s High Performance Driving Event.  No warranties or guarantees are implied or expressed by the passing of the inspection performed.  I acknowledge that the safe condition and operation of my Car is entirely my responsibility.  I take full and sole responsibility for any vehicle problems, malfunctions, or damage that may occur in connection with the operation or performance of my Car prior to, during, or subsequent to the Event.  

 

Driver #1 Signature: ____________________________________ Date: _________________
Driver #2 Signature: __________________________________________________________

Minimum Standard Grid Tech List: 
_____1. No loose objects or distracting items inside car

_____2. At least one functioning brake light


_____3. Gas cap & battery secure

_____4. Wheel lug nuts torque to specification 
_____5. Helmet with proper Snell rating

_____6. Proper clothing attire per regional rules – Closed toe shoes; no lug-sole shoes/boots
_____7. Car numbers in place
_____8. Tires must be in good condition with no cracks and/or cuts
_____9. Sufficient brake pad thickness

